- Achiziţii de date cu ajutorul unei interfeţe SOUNDBLASTER

Autor: sani

- Intoducere

În acest document este descris un program în C/C++ de achiziţii de date cu ajutorul unei interfeţe SOUNDBLASTER, şi prelucrarea semnalelor care permit:

- calculul funcţiilor de autocorelaţie

- calculul funcţiilor de intercorelaţie

- Principiul funcţionării

Programul foloseşte o placa SOUNDLBLASTER pentru achiziţionarea semnalelor, semnalul analogic (continuu) este convertat în semnal digital (discret), care este prelucrat de program care realizează funcţiile cerute.

- Cerinţe

- Calculator >386

OS: DOS >6.20 (Win9x,Win XP,NT,2000)

Placa SOUNDBLASTER

Monitor VGA

Programul a fost testat pe un sistem cu OS: Win98, SVGA, placa de sunet C-Media Inc.8330 (compatibil SB16), pe acest sistem programul funcţionează stabil.

- Descriere program

Programul principal 123.exe,utilizează biblioteca grafică egavga.bgi.Are ca argument 0,1,2:

- 0 – doar osciloscop

- 1 – osciloscop + fct. de autocorelaţie

- 2 – osciloscop +fct. de intercorelaţie

În program sunt utilizate:

IRQ,DMA,VGAPAGEFLIP.etc

Programul foloseşte o placa SOUNDBLASTER pentru achiziţionarea semnalelor

Semnalul analogic este aplicat la intrarea de microfon a plăcii SOUNDBLASTER.

Semnalul analogic (continuu) este convertat în semnal digital (discret) de CAN din placa SOUNDBLASTER. Structura principial este reprezentat în Figura 1:

Programul foloseşte acest convertor analog-digital pe 8 biti.(În cazul SB16 acest convertor este pe 16 biţi dar se poate folosi şi pe 8 biţi)

Transferul de date se face prin DMA.

Pentru a funcţiona corect pentru placa SOUNDBLASTER trebuie să fie setat următorii parametrii:

- Adresa de bază 0x220

- IRQ 5

- DMA 1

Programul nu detectează automat setările, dacă placa SOUNDBLASTER are alte parametrii trebuie modificat fişierul sbaster.h,dma_mem.c

Programarea unităţii DSP (Digital Signal Processor) din placa SOUNDBLASTER,se face prin porturi.

Figura 1:

Porturile plăcii SOUDBLASTER

- Tabel 1:

02x00h         C/MS 1-6 - Data Port                    Write       SB Only

02x00h         FM Music - Left Status Port             Read        SBPro

02x00h         FM Music - Left Register Port           Write       SBPro

02x01h         C/MS 1-6 - Register Port                Write       SB Only

02x01h         FM Music - Left Data Register           Write       SBPro

02x02h         C/MS7-12 - Data Port                    Write       SB Only

02x02h         FM Music - Right Status Port            Read        SBPro

02x02h         FM Music - Right Register Port          Write       SBPro

02x03h         C/MS7-12 - Register Port                Write       SB Only

02x03h         FM Music - Right Data Register          Write       SBPro

02x04h         Mixer - Register Port                   Write       SBPro

02x05h         Mixer - Data Register                   Read/Write  SBPro

02x06h         DSP - Reset                             Write       SB

02x08h         FM Music - Compatible Status Port       Read        SB

02x08h         FM Music - Compatible Register Port     Write       SB

02x09h         FM Music - Compatible Data Register     Write       SB

02x0Ah         DSP - Read Data                         Read        SB

02x0Ch         DSP - Write Data or Command             Write       SB

02x0Ch         DSP - Write Buffer Status               Read        SB

02x0Dh         DSP - Timer Interrupt Clear             Read        SB16???

02x0Eh         DSP - Data Available Status             Read        SB

02x0Eh         DSP - IRQ Acknowledge, 8-bit            Read        SB

02x0Fh         DSP - IRQ Acknowledge, 16-bit           Read        SB16

02x10h         CD-ROM - Data Register                  Read        SBPro

02x10h         CD-ROM - Command Port                   Write       SBPro

02x11h         CD-ROM - Status Port                    Read        SBPro

02x12h         CD-ROM - Reset                          Write       SBPro

02x13h         CD-ROM - Enable                         Write       SBPro

0388h          AdLib - Status Port                     Read        SB

0388h          AdLib - Register Port                   Write       SB

0389h          AdLib - Data Register                   Write       SB

038Ah          Advanced AdLib - Status Port            Read        SB16

038Ah          Advanced AdLib - Register Port          Write       SB16

038Bh          Advanced AdLib - Data Register          Write       SB16

03x00h         MPU-401 - Data Port                     Read/Write  SB16???

03x01h         MPU-401 - Status Port                   Read        SB16???

03x01h         MPU-401 - Command Port                  Write       SB16???

0200h-0207h    Joystick                                Varies      SB

- Comenzi DSP

- Tabel 2:

003h           ASP Status                                          SB16ASP

004h           DSP Status (Obsolete)                               SB2.0-Pro2

004h           ASP ???                                             SB16ASP

005h           ASP ???                                             SB16ASP

010h           Direct DAC, 8-bit                                   SB

014h           DMA DAC, 8-bit                                      SB

016h           DMA DAC, 2-bit ADPCM                                SB

017h           DMA DAC, 2-bit ADPCM Reference                      SB

01Ch           Auto-Initialize DMA DAC, 8-bit                      SB2.0

01Fh           Auto-Initialize DMA DAC, 2-bit ADPCM Reference      SB2.0

020h           Direct ADC, 8-bit                                   SB

024h           DMA ADC, 8-bit                                      SB

028h           Direct ADC, 8-bit (Burst)                           SB-Pro2

02Ch           Auto-Initialize DMA ADC, 8-bit                      SB2.0

030h           MIDI Read Poll                                      SB

031h           MIDI Read Interrupt                                 SB

032h           MIDI Read Timestamp Poll                            SB???

033h           MIDI Read Timestamp Interrupt                       SB???

034h           MIDI Read Poll + Write Poll (UART)                  SB2.0

035h           MIDI Read Interrupt + Write Poll (UART)             SB2.0???

037h           MIDI Read Timestamp Interrupt + Write Poll (UART)   SB2.0???

038h           MIDI Write Poll                                     SB

040h           Set Time Constant                                   SB

041h           Set Sample Rate                                     SB16

045h           Continue Auto-Initialize DMA, 8-bit                 SB16

047h           Continue Auto-Initialize DMA, 16-bit                SB16

048h           Set DMA Block Size                                  SB2.0

074h           DMA DAC, 4-bit ADPCM                                SB

075h           DMA DAC, 4-bit ADPCM Reference                      SB

076h           DMA DAC, 2.6-bit ADPCM                              SB

077h           DMA DAC, 2.6-bit ADPCM Reference                    SB

07Dh           Auto-Initialize DMA DAC, 4-bit ADPCM Reference      SB2.0

07Fh           Auto-Initialize DMA DAC, 2.6-bit ADPCM Reference    SB2.0

080h           Silence DAC                                         SB

090h           Auto-Initialize DMA DAC, 8-bit (High Speed)         SB2.0-Pro2

098h           Auto-Initialize DMA ADC, 8-bit (High Speed)         SB2.0-Pro2

0A0h           Disable Stereo Input Mode                           SBPro Only

0A8h           Enable Stereo Input Mode                            SBPro Only

0Bxh/0Cxh      Generic DAC/ADC DMA (16-bit, 8-bit)                 SB16

0D0h           Halt DMA Operation, 8-bit                           SB

0D1h           Enable Speaker                                      SB

0D3h           Disable Speaker                                     SB

0D4h           Continue DMA Operation, 8-bit                       SB

0D5h           Halt DMA Operation, 16-bit                          SB16

0D6h           Continue DMA Operation, 16-bit                      SB16

0D8h           Speaker Status                                      SB

0D9h           Exit Auto-Initialize DMA Operation, 16-bit          SB16

0DAh           Exit Auto-Initialize DMA Operation, 8-bit           SB2.0

0E0h           DSP Identification                                  SB2.0

0E1h           DSP Version                                         SB

0E3h           DSP Copyright                                       SBPro2???

0E4h           Write Test Register                                 SB2.0

0E8h           Read Test Register                                  SB2.0

0F0h           Sine Generator                                      SB

0F1h           DSP Auxiliary Status (Obsolete)                     SB-Pro2

0F2h           IRQ Request, 8-bit                                  SB

0F3h           IRQ Request, 16-bit                                 SB16

0FBh           DSP Status                                          SB16

0FCh           DSP Auxiliary Status                                SB16

0FDh           DSP Command Status                                  SB16

- Programul foloseşte următoarele registre:

02x06h         DSP – Reset

02x0Ah         DSP - Read Data

02x0Ch         DSP - Write Data or Command

02x0Ch         DSP - Write Buffer Status

02x0Dh         DSP - Timer Interrupt Clear

02x0Eh         DSP - Data Available Status

Comenzi DSP:

014h           DMA DAC, 8-bit

024h           DMA ADC, 8-bit

040h           Set Time Constant

0D1h           Enable Speaker

0D3h           Disable Speaker

02x06h     DSP - Reset

Descriere

Resetează DSPul, terminând toate operaţiile anterioare.

- Procedura

- Scrie 001h

- Aşteaptă 3.3us minimum

- Scrie 000h

- şteaptă 100us maximum pentru DSP Data Available(02x0Eh)

- Citire 0xAA de la DSP Read Data (02x0Ah)

02x0Ah     DSP - Read Data

Descriere

Portul de intrare pentru citirea DSP octet.

- Procedura

- Aşteapta până când bitul 7=1 de la DSP Data Abailable Status(02x0Eh read)

- Citire octet de pe DSP Read Data port (02x0Ah read)

02x0Ch     DSP - Write Data or Command

Descriere

Portul de iesire pentru comenzi DSP si pentru date.

- Procedura

- Aşteaptă până când bitul 7=1 de la DSP Write Buffer Status (02x0Ch read)

- Scriere octet la DSP Write Data or Command port (02x0Ch write)

02x0Ch     DSP - Write Buffer Status

Descriere

Indică dacă DSPul poate să primeşte date prin DSP Write Data or Command port (02x0Ch write).

- Procedura

- Aşteaptă până când bitul 7=1 de la DSP Write Buffer Status (02x0Ch read)

- Scriere octet la DSP Write Data or Command port (02x0Ch write)

02x0Eh     DSP - Data Available Status

Descriere

Indică dacă DSPul are date disponibile pe portul DSP Read Data port (02x0Ah read).

- Procedura

- Aşteapta până când bitul 7=1 de la DSP Data Abailable Status(02x0Eh read)

- Citire octet de pe DSP Read Data port (02x0Ah read)

- Comenzi DSP

014h       DMA DAC, 8-bit

- COMMAND->LENGTHLOBYTE->LENGTHHIBYTE

- Descriere

Iniţializează transfer DMA pe 8 biţi.

- Procedura

- Instalare procedura de întrerupere,actualizare masca pic.

- Setare constanta de timp,sau timpul de eşantionare (Set Time Constant(040h))

- Comanda Enable Speaker (0D1h)

- Setare controlerul DMA (mode = 048h + channel)

- Comanda DMA DAC,8-bit (014h)

- Acknowledge IRQ PIC & SB

- Comanda Disable Speaker (0D3h)

- LENGTH = SAMPLES - 1

040h       Set Time Constant

- COMMAND->TIMEBYTE

- Descriere

Seteză rate de eşantionare

- Formula

TimeConstant = 256 - (1000000 / (SampleChannels * SampleRate))

0D1h       Enable Speaker

- COMMAND

- Descriere

Activeazâ ieşirea

0D3h       Disable Speaker

- COMMAND

- Descriere

Dezactivează ieşirea

Funcţii legate de placa SOUNDBLASTER se găsesc în fişierul sblaster.c

#ifndef SBH

#include "sblaste.h"

#endif

unsigned char sbdmaon;

void interrupt(*oldintvector)(void);

void interrupt SB_Handler(void);

void interrupt SB_Handler(void){

sbdmaon=0;

inp(DSP_IRQAck8);

outp(INT_Controller,PIC_IRQAck);

}

void DSP_Write(unsigned char a){

while (inp(DSP_WriteBufferStatus) & 0x80); //bit7=0 ->ready;

outp(DSP_WriteDataOrCommand,a);

}

unsigned char DSP_Read(void){

long timeout=10000L;

- //bit 7=1->ready

while( (!inp(DSP_DataAvailableStatus)&0x80) && (timeout--)  );

return inp(DSP_ReadData);

}

unsigned char DSP_Reset(void){

unsigned char i=100;

outp(DSP_ResetPort,1);

outp(DSP_ResetPort,0);

while((DSP_Read()!=0xAA) && (i--) );

return i;

}

void SB_InstallHandler(void){

disable();

outp(PIC_Maskport, (inp(PIC_Maskport)|IRQ_Stop));

oldintvector = getvect(IRQ_Intvector);

setvect(IRQ_Intvector, SB_Handler);

outp(PIC_Maskport, (inp(PIC_Maskport)&IRQ_Start));

enable();

}

void SB_UninstallHandler(void){

disable();

outp(PIC_Maskport, (inp(PIC_Maskport)|IRQ_Stop));

setvect(IRQ_Intvector, oldintvector);

enable();

}

Controlerul DMA

Controlerul DMA este folosit pentru transfer de date între echipamente de intrare/iesire (porturi I/O) şi memorie, fără intervenţia procesorului. Este folosit în general de catre unităţi de disk, hard diskuri, dar poate fi folosit pentru orice port I/O cât timp nu interferează cu alte unităţi standard.

Setarea controlerului DMA pentru transfer DMA

- Activare masca canal

- Stergere byte pointer

- Configurare modul de transfer

- Scriere adresa de pagina

- Scriere adresa de deplasament (offset)

- Scriere lungime-1

- Deactivare masca canal

Adresa DMA este pe 20 biţi, această adresă este calculată în program.Registrul de adresa de pagina este setat cu o valoare pe 4 biţi care reprezintâ biţii 16-19 din adresa DMA de 20 biţi.Memoria alocata transferului DMA trebuie să încapă pe o pagina de 64Koctet. În program această condiţie este satisfăcut alocând de două ori mai multă memorie(în total max 64Koctet ), pentru transferul DMA, iar dacă aceasta zomă de memorie nu încapă pe o pagină de 64Kocteţi.Adresa de început este mărit pînă la începutul paginii 2. Astfel zona de memorie sigur, încapă pe pagina aceasta.

Despre controlerul DMA

The original PC supports four 8-bit DMA channels, across a 20-bit address

space, using an Intel 8237A DMA controller chip.  The AT supports seven

DMA channels by cascading a second 8237A DMA controller.  The differences

between PC and AT DMA are covered at the end of this section.

Channel  Usage in PC and XT

_______  ____________________________________

0     memory refresh    (highest priority)

1     not used

2     diskette adapter

3     hard disk adapter (lowest priority)

Note: All PC-compatibles use DMA channel 0 for memory refresh.  If

memory is not refreshed as set by the BIOS, data in RAM will

degrade within a few hundredths of a second.  It is wise to avoid

monkeying with DMA port settings.

Port  Description

____  ______________________________________________________________________

000H-007H DMA base address an offset registers

All are 16-bit registers: read/write the low byte, then the high byte

at the same I/O port.  Base addresses are offsets from a DMA Page

(see below).

000H  Write: DMA channel 0 base address (also sets current address)

Read:  DMA channel 0 current address

001H  Write: DMA channel 0 base address and word count

Read:  DMA channel 0 current word count

002H  Write: DMA channel 1 base address

Read:  DMA channel 1 current address

003H  Write: DMA channel 1 base address and word count

Read:  DMA channel 1 current word count

004H  Write: DMA channel 2 base address           (diskette adapter)

Read:  DMA channel 2 current address              "

005H  Write: DMA channel 2 base address and word count  "

Read:  DMA channel 2 current word count           "

006H  Write: DMA channel 3 base address          (hard disk adapter)

Read:  DMA channel 3 current address              "

007H  Write: DMA channel 3 base address and word count  "

Read:  DMA channel 3 current word count           "

---- -----------------------------------------------------------------------

008H-00fH DMA control/status registers

008H  Write: DMA command register

+7-6-5-4-3-2-1-0+

¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦

+---------------+ bit

¦ ¦ ¦ ¦ ¦ ¦ ¦ +- 0: 1=enable memory-to-memory DMA (ch0-ch1)

¦ ¦ ¦ ¦ ¦ ¦ +--- 1: 1=enable Ch0 address hold

¦ ¦ ¦ ¦ ¦ +----- 2: 1=disable controller

¦ ¦ ¦ ¦ +------- 3: 1=select compressed timing mode

¦ ¦ ¦ +--------- 4: 1=enable rotating priority

¦ ¦ +----------- 5: 1=select extended write mode; 0=late write

¦ +------------- 6: 1=select DRQ sensing as active high; 0=low

+--------------- 7: 1=select DACK sensing as active high; 0=low

Read:  DMA status register

+7-6-5-4-3-2-1-0+

¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦

+---------------+ bit

+-----+ +------- 0-3: channel 0-3 has reached terminal count

+------------ 4-7: channel 0-3 has a request pending

009H  Write: request register

+7-6-5-4-3-2-1-0+

¦  unused ¦ ¦   ¦

+---------------+ bit

¦ +--- 0-1: select channel (00=0; 01=1; 10=2; 11=3)

+-----   2: 1=set request bit for channel; 0=reset request

00aH  Write: single mask bit register

+7-6-5-4-3-2-1-0+

¦  unused ¦ ¦   ¦

+---------------+ bit

¦ +--- 0-1: select channel (00=0; 01=1; 10=2; 11=3)

+-----   2: 1=set mask for channel; 0=clear mask (enable)

00bH  Write: mode register

+7-6-5-4-3-2-1-0+

¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦

+---------------+ bit

+-+ ¦ ¦ +-+ +--- 0-1: select channel (00=0; 01=1; 10=2; 11=3)

¦  ¦ ¦  +------ 2-3: xfer type: 00=verify (nop); 01=write; 10=read

¦  ¦ +---------   4: 1=enable auto-initialization

¦  +-----------   5: 1=select addr increment; 0=address decrement

+-------------- 6-7: 00=demand mode; 01=single

10=block; 11=cascade

00cH  Write: clear byte pointer flip-flop.  Any write clears the flip-flop

so that the next write to any of the 16-bit registers is

decoded as the low byte.  The next is the high byte, then next

is low, etc.

00dH  Write: master clear.  Any OUT clears the ctrlr (must be re-init'd)

Read:  temporary reg.  Last byte in memory-to-memory xfer (not used)

00eH  Write: Clear mask registers.  Any OUT enables all 4 channels.

00fH  Write: master clear.  Clear or mask any or all of the channels.

+7-6-5-4-3-2-1-0+

¦       ¦ ¦ ¦ ¦ ¦

+---------------+ bit

¦ ¦ ¦ +- 0: 1=mask channel 0; 0=enable

¦ ¦ +--- 1: 1=mask channel 1;

¦ +----- 2: 1=mask channel 2;

+------- 3: 1=mask channel 3;

Read:  temporary reg.  Last byte in memory-to-memory xfer (not used)

081H-08fH DMA page registers.

To select a starting address for a DMA operation, do an OUT to the

page register (ports 81H-83H) for the selected channel then set the

base address (ports 00H-07H) for the channel.  A page register is set

with a 4-bit value that represents bits 16-19 of the 20-bit DMA

address.  Since the current address is a 16-bit value, it is not

possible to cross a 64K boundary (e.g., address 1000:0, 2000:0, etc.)

with a DMA operation.

081H  Channel 2 page register  (diskette DMA)

082H  Channel 3 page register  (hard disk DMA)

083H  Channel 1 page register

____________________________________________________________________________

_¦AT DMA__

The DMA system on the AT is basically upwardly-compatible with PC and XT

DMA.  In addition to the four 8-bit channels of the PC, the AT adds a

second 8237A-5 DMA controller which supports channels 4-7.

Channel  Usage in AT

_______  ________________________________________________________________

0     spare                                   -+

1     SDLC (Synchronous Data Link Control)     ¦- 8-bit DMA channels

2     diskette adapter                         ¦

3     hard disk adapter                       -+

4     (controller 2) cascade for controller 1 -+

5     spare                                    ¦- 16-bit DMA channels

6     spare                                    ¦

7     spare                                   -+

---- -----------------------------------------------------------------------

081H-08fH DMA page registers.  On the AT, all 8 bits of the Page registers

are used.  They become the high 8-bits of a 24-bit address space

(with the low 16-bits being set in a channel's base/current address

register). The page size is 128K (64K words) so DMA transfers must not

cross a 128K boundary (e.g., address 2000:0, 4000:0, 6000:0, etc.)

081H  Channel 2 page register  (diskette DMA)  (address bits 16-23)

082H  Channel 3 page register  (hard disk DMA) (address bits 16-23)

083H  Channel 1 page register                  (address bits 16-23)

087H  Channel 0 page register                  (address bits 16-23)

089H  Channel 6 page register  (address bits 17-23)

08bH  Channel 5 page register  (address bits 17-23)

08aH  Channel 7 page register  (address bits 17-23)

08fH  refresh

---- -----------------------------------------------------------------------

0c0H-0dfH AT DMA controller registers for 16-bit DMA I/O.  Channels 0-3 work

with 8-bit I/O as in the PC.  Additional channels 4-7 support 16-bit

device-to-memory and memory-to-device transfers.  Transfers for these

channels always start on a word boundary and all addresses and counts

are for 16-bit words (e.g., a base address of 123H actually refers to

offset 246H from the page for that channel).

0c0H  Channel 4 base and current address (bits 1-16; bit 0 assumed 0)

0c2H  Channel 4 current word count

0c4H  Channel 5 base and current address (bits 1-16)

0c6H  Channel 5 current word count

0c8H  Channel 6 base and current address (bits 1-16)

0caH  Channel 6 current word count

0ccH  Channel 7 base and current address (bits 1-16)

0ceH  Channel 7 current word count

---- -----------------------------------------------------------------------

0d0H-0dfH AT DMA control/status registers

0d0H  Write: command register

Read:  status register

0d2H  Write request register

0d4H  Write single mask register bit

0d6H  Write Mode register

0d8H  Clear byte pointer flip-flop

0daH  Write: master clear

Read:  temporary register

0dcH  Clear mask register

0deH  Write all mask register bits

---- -----------------------------------------------------------------------

Funcţii legate de controlerul DMA se găsesc în fişierul dma_mem.c.

//all functions for 8 bit dma

#define getlinearaddr(p) ((unsigned long)FP_SEG(p)*16 + (unsigned long)FP_OFF(p))

int DMA_AllocMem(unsigned long size){

unsigned long ap;

dmabuffer=farmalloc(2*size);

if(dmabuffer==NULL)return 0;

dmaptr = (unsigned char far *)dmabuffer;

if( (( getlinearaddr(dmaptr) % 65536L) + size )> 65536L)

{

//dma 64k block override

// dmaptr +=size;

buf_addr=getlinearaddr(dmaptr);

buf_addr+=0x0FFFFL;

buf_addr&=0xF0000L;

ap=(buf_addr>>16)&15;

dmaptr=MK_FP(((unsigned short)ap<<12)&0xF000,0);

printf("->Second DMA buf choosed<-");

}

else printf("->FIRST DMA buf choosed<-");

dmaptr1=dmaptr;

buf_addr = getlinearaddr(dmaptr);

buf_page = (unsigned char)((buf_addr >> 16)&0x000F);

buf_ofs  = (unsigned int)((buf_addr) % 65536L);

printf("\nDMA buf size:        %8.0f",(float)size);

printf("\nbuf_adr              %8.0f",(float)buf_addr);

printf("\nbuf_page             %8.0f",(float)buf_page);

printf("\nbuf_ofs              %8.0f",(float)buf_ofs);

return 1;

}

void DMA_DAC8Start(void){

sbdmaon=1;

/* Program DMA controller */

outp(DMA_MaskPort,	SB_DMA+4);	//sn dma 0..3

outp(DMA_ClearFlipFlop,   0x00);	// -lo -hi

outp(DMA_ModePort,     DMA_WantedMode);

outp(DMA_BaseAddrPort, LO(buf_ofs));

outp(DMA_BaseAddrPort, HI(buf_ofs));

outp(DMA_CountPort,    LO(DMA_Length-1));

outp(DMA_CountPort,    HI(DMA_Length-1));

outp(DMA_PagePort,     buf_page);

outp(DMA_MaskPort,     SB_DMA);

//Program the SB

//set time constant

DSP_Write(DSPcm_SetTimeConstant);

DSP_Write(245);//165  //245 //210

//use 8 bit mono no aoutoinit

DSP_Write(DSPcm_DMADAC8);

DSP_Write(LO(DMA_Length-1));

DSP_Write(HI(DMA_Length-1));

//!! TRANSFER STARTS NOW !! --------------------------

}

void DMA_ADC8Start ( unsigned short len )

// len = number of bytes to record to unsigned char *aligned (<=65000)

{

len--;

sbdmaon=1;

// tc = time constant = 256L - (1000000UL/samples per second)

//    inp(0x022E);

//    DSP_Write(0x40);

//    DSP_Write(245);

DSP_Write(DSPcm_SetTimeConstant);

DSP_Write(245);//165  //245 //210

outp(DMA_MaskPort,	5);	//sn dma 0..3

outp(DMA_ClearFlipFlop,   0x00);	// -lo -hi

outp(DMA_ModePort,     0x45);

outp(DMA_BaseAddrPort, LO(buf_ofs));

outp(DMA_BaseAddrPort, HI(buf_ofs));

outp(DMA_CountPort,    LO(len));

outp(DMA_CountPort,    HI(len));

outp(DMA_PagePort,     buf_page);

outp(DMA_MaskPort,	1);	//sn dma 0..3

DSP_Write(0x24);

DSP_Write(LO(len));

DSP_Write(HI(len));

}

void DMA_FreeMem(void){

farfree(dmabuffer);

}

Funcţiile de intercorelaţie şi de autocorelaţie

Funcţia de autocorelaţie

(1)

Funcţia de intercorelaţieϕxx(τ)=limT−∞1ToTx(t)x(t+τ)dtϕxy(τ)=limT−∞1ToTx(t)y(t+τ)dt

ϕxx(n)=1Nk=1Nx(k)x(k+n)ϕxy(n)=1Nk=1Nx(k)y(k+n)

(3)

În program sunt implementate  funcţiile (2) şi (4) pentru n>=0.

(2)

(4)

Funcţiile legate de analiza semnalelor ( funcţia de autocorelaţie şi intercorelaţie ) se află în fişierul analysis.c

void Autocorrelation(unsigned char *adat,

unsigned int nn,

int x,

int y,

char param){

unsigned char b;

unsigned char *ptr,*pxn,*pxn2;

unsigned char xk,xn;

double suma=0;

const per=2;

int maxim=0;

int vx=x,vy=y,vy_o=0;

int k,n;

if(param) {fliprectangle(x-1-1,y-127/per-1,x+nn+1,y+128/per+1);return;}

ptr=adat;

pxn=adat;

for(n=0;n<nn;n++)

{

//suma

pxn++;

pxn2=pxn;

ptr=adat;

vx++;

suma=0;

for(k=1;k<nn;k++){

xk=*ptr;

ptr++;

if(n+k<nn){

xn=*pxn2;

pxn2++;

}

else xn=127;

suma+=(xk-127)*(xn-127);

}//k

suma/=nn;

vy=(int)(suma/5);

if(vy>127/per)vy=127/per;

if(vy<-127/per)vy=-127/per;

if(vy>maxim)maxim=vy;

//  putpixel(vx,y+vy,15);

setcolor(15);

line(vx,-vy+y,vx-1,-vy_o+y);

vy_o=vy;

}//n

}

void Intercorrelation(unsigned char *adat,

unsigned int nn,

int x,

int y,

char param){

unsigned char b;

unsigned char *ptr,*pxn,*pxn2;

unsigned char xk,xn;

double suma=0;

const per=2;

int maxim=0;

int vx=x,vy=y,vy_o=0;

int k,n;

double t=0;

double f=200;

double A=80;

double jel;

ptr=adat;

pxn=adat;

if(param) {fliprectangle(x-1-1,y-127/per-1,x+nn+1,y+128/per+1);return;}

for(n=0;n<nn;n++)

{

//suma

ptr=adat;

vx++;

suma=0;

for(k=1;k<nn;k++){

xk=*ptr;

ptr++;

if(n+k<nn){

//sinus 800hz 7ns 0 - nn

//		t=((n+1)/nn)*(7/1000);

t=(dcount*((n+k)+0.1))/((nn+0.1)*1000);

jel=(A*sin(2*3.1416*f*t));

xn=(char)(jel)/per;

}

else xn=0;

suma+=(xk-127)*(xn);

}//k

suma/=nn;

vy=(int)(suma/50);

if(vy>127/per)vy=127/per;

if(vy<-127/per)vy=-127/per;

if(vy>maxim)maxim=vy;

setcolor(15);

line(vx,-vy+y,vx-1,-vy_o+y);

vy_o=vy;

}//n

}

Fişierele programului

123.c –program principal

sblaster.h

sblaster.c

mt.c

analysis.c

dma_mem.c

fileio.c

Nu toate fişierele şi funcţiile sunt folosite de programul final din aceste fişiere. Unele funcţii au fost folosite în etapa scrierii, testării programului.

Variabile din program

VGAFLIPPAGE – afişare folosind două pagini VGA (se poate să nu funcţioneze la unele placi VGA)

DRAFTMODE – afişare simplificată pentru imprimantă

Imagini din program

Len=14  Lungimea semnalului din grafic în milisecunde.În acest caz 14ms.

5  -Cadre afişate pe secundă (fps)

Bibliografie

[1] Cursuri

[2] Internet

[3] TechHelp